|  | Inspection | Maintenance |
| --- | --- | --- |
| Deck | clean, uncluttered | Non-slip wax |
| Deck core | de-lamination, water penetration – flex , oil-canning, blisters | Fiberglass rebuild if serious |
| Cleats / cam cleats | corrosion, leaks, alignment | Remove, clean, replace |
| Chainplates | corrosion, leaks, alignment | Remove, clean, replace |
| Pulpit | corrosion, leaks, alignment | Remove, clean, replace |
| Mast towing hardware | corrosion, leaks, alignment | Remove, clean, replace |
| Pushpit | corrosion, leaks, alignment | Remove, clean, replace |
| Mast towing hardware | corrosion, leaks, alignment | Remove, clean, replace |
| Stanchions | corrosion, leaks, alignment | Remove, clean, replace |
| Fairleads | corrosion, leaks, alignment | Remove, clean, replace |
| Winches | corrosion, leaks, alignment | Open, service, lube, replace |
| Rails | corrosion, leaks, alignment, pins and retaining rings | Remove, clean, replace |
| Lifelines | corrosion, leaks, alignment, pins and retaining rings, tension | Remove, clean, replace, re-tension |
| Forestay(s) | fish hooks, chafing, alignment, pins and retaining rings, attachments | Remove, clean, replace, re-tension |
| Backstay | fish hooks, chafing, alignment, pins and retaining rings, attachments | Remove, clean, replace, re-tension |
| Upper shrouds | fish hooks, chafing, alignment, pins and retaining rings, attachments | Remove, clean, replace, re-tension |
| Lower shrouds | fish hooks, chafing, alignment, pins and retaining rings, attachments | Remove, clean, replace, re-tension |
| Blocks / pulleys / travelers / alignment hardware | straight, even, corrosion, damage, wear, alignment, pins and retaining rings | Remove, clean, replace, re-tension |
| Rope halyards | chafing | Remove, replace, re-tension |
| Rope-wire connections | damage | Remove, replace, re-tension |
| Wood trim | UV damage, fasteners | Oil, re-fasten, replace |
| Mast crutch | UV damage, fasteners | Oil, re-fasten, replace |
| Windows | sealed, fasteners, clean, leaks | Remove, clean, re-caulk, re-fasten |
| Zincs | wear | Remove, replace |
| Rub rails | straight, fastened, damage, leaks | Remove, inspect, re-caulk, replace |
| Anti-fouling paint | Holidays, scrapes | touch-up |
| Hatches | leaks, cracks, seals, clamps | Remove, or replace |
| Lockers | leaks, cracks, seals, clamps | Remove, inspect, repair or replace |
| Lazarette | leaks, cracks, seals, clamps | Remove, inspect, repair or replace |
| Transom | leaks, cracks | inspect, repair |
| Mast | straight, even, corrosion, damage, wear | Clean, re-tension, repair or replace |
| Mast light wire and deck socket | Corrosion, damage | Clean, service, repair or replace |
| Mast hinge | Corrosion, damage | Clean, service, repair or replace |
| Mast hinge pin / pivot pin – lock nut | Corrosion, damage | Clean, service, repair or replace |
| Spreaders | straight, corrosion, damage, wear, attached to shroud line, end protection | Clean, service, repair or replace |
| Boom | straight, corrosion, damage, wear | Clean, service, repair or replace |
| Mainsail cleat | straight, corrosion, damage, wear | Clean, service, repair or replace |
| Connection to mast | corrosion, damage, wear | Clean, service, repair or replace |
| Boom vang | Cracks, alignment | Clean, service, repair or replace |
| Poles | straight, corrosion, damage, wear | Clean, service, repair or replace |
| Sail tracks | fasteners present, secure, leaks | Clean, service, repair or replace |
| Welds | Cracks, corrosion, leaks | Clean, inspect, repair or replace |
| Navigation lights | Work, damage, dirty | Clean, repair or replace |
| Bumpers | Damaged, dirty | Clean, repair or replace |
| Dock lines | Chafing, dirty | Clean, repair or replace |
| Canvas |  |  |
| Dodgers | Chafing, dirty, worn | Clean, service, repair or replace |
| Biminis | Chafing, dirty, worn | Clean, service, repair or replace |
| Equipment covers | Chafing, dirty, worn | Clean, service, repair or replace |
| Pop-top cover | Chafing, dirty, worn | Clean, service, repair or replace |
| Cabin |  |  |
| Floor | damage, leaks | Clean, service, repair or replace |
| Ceiling liner | damage, leaks | Clean, service, repair or replace |
| Bulkheads / dividers | not shifted at connections to deck hull, or ceiling liner, water damage | Secure, repair or replace |
| Backing plates for deck hardware | Adequate, straight, corrosion, damage, wear |  |
| Galley |  |  |
| Pipes | work, leaks, clamps | repair or replace |
| electrical wiring | Work, secure | Repair or replace |
| fuel tanks | Work, full, secure | Fill, Repair or replace |
| pumps | Work, leaks | Repair or replace |
| Dinette | Secure, damage | Repair or replace |
| Water tanks | Leaks, secure | Repair or replace |
| Dry storage | Secure, damage | Repair or replace |
| lights | Work, corrosion | Repair or replace |
| Berths |  | Clean, service, repair |
| Storage areas | Dry, secure | Clean, service, repair |
| Lights | Work, corrosion | Clean, service, repair or replace |
| Bilge | Oil, debris | clean |
| Pumps | operational, leaks | Repair or replace, tighten clamps |
| Limber holes | debris | clear |
| Hull |  |  |
| Blisters | Drain, inspect if water in laminate | Remove, repair, restore gel coat |
| Cracks | Inspect if beyond gel coat | Fill, restore gel coat |
| Scratches | Inspect if beyond gel coat | restore gel coat |
| Impact damage | Inspect if beyond gel coat | restore gel coat |
| Thru-hull fittings | corrosion, leaks | double clamps, lube seacocks |
| Cockpit drains | Drains, leaks |  |
| Outboard well | Drains, leaks |  |
| Water ballast valve | Leaks, corrosion | Clean, replace gaskets, hardware |
| Pop-top |  |  |
| Locking devices | alignment, functions | Repair or replace |
| Seal | straight, flexible, continuous | Repair or replace |
| Forward hatch |  |  |
| Locking clamp | alignment, functions | Repair or replace |
| Seal | straight, flexible, continuous | Repair or replace |
| Cabin hatches |  |  |
| Seal | straight, flexible, continuous | Repair or replace |
| Keel / centerboard | Damage – leading edge | Repair or replace |
| Lock pin / pivot pin | alignment, corrosion, damage, wear | Repair or replace |
| Uphaul hardware | alignment, corrosion, damage, wear | Repair or replace |
| Rudder |  | Repair or replace |
| Blade | damage, leading edge | Repair or replace |
| Hinges, pintels | damage, distortion, fit, guide holes round not oblong | Lube, repair or replace e, |
| Pivot bolt and nut | damage, distortion, fit, guide holes round not oblong | Lube, repair or replace |
| Tiller | blisters, de-lamination, damage, distortion | Repair or replace |
| Engine |  |  |
| Spark plugs | Present, working | Clean, replace |
| zincs | wear | Replace |
| impeller | works | Replace |
| primer bulb | Works, leaks | Clean, repair or replace |
| fuel line | Works, leaks | Clean, repair or replace |
| Fuel tank | Clean, leaks, gauges, | Clean, fill, repair or replace |
| oil | Clean, full | Replace |
| oil filter | Works, leaks | repair or replace |
| fuel filter | Works, leaks | repair or replace |
| Propellers | bent, nicked, notched, cracked | Replace |
| Backfire flame control device | present | replace |
| Safety cable | sound | repair or replace |
| Clamps | Tight, lock, motor solid | replace |
| Papers |  |  |
| Title | Match to hull numbers | Inquiry, report, no-go |
| Insurance | Match to hull numbers | Inquiry, report, no-go |
| Registration | Match to hull numbers | Inquiry, report, no-go |
| HIN | Match to trailer registration | Inquiry, report, no-go |
| manufacturer plate | Match to trailer registration | Inquiry, report, no-go |
| Coast Guard required equipment |  |  |
| PFDs | inspect | replace as needed |
| Fire extinguishers | inspect, check expiration date | replace as needed |
| Flares | inspect, check expiration date | replace as needed |
| Air Horn / hailer | inspect | Repair or replace |
| Nav lights | inspect | Repair or replace |
| Dinghy / life raft | inspect | Repair or replace |
| Placards | : oil discharge, waste disposal | Repair or replace |
| Other |  |  |
| Anchors | inspect | Repair or replace |
| Anchor rode[s], chain, connectors | inspect | Repair or replace |
| Sea anchor | inspect | Repair or replace |
| Series drogue | inspect | Repair or replace |
| Rocker stopper | inspect | Repair or replace |
| Jack lines | inspect | Repair or replace |
| Safety harness(es) | inspect | Repair or replace |
| Man overboard float, flag, rescue rope | inspect | Repair or replace |
| Radar reflector | inspect | Repair or replace |
| Mast float | inspect | Repair or replace |
| At-anchor lights | inspect | Repair or replace |
| Electrical |  |  |
| Batteries | water level if not sealed, terminals, terminal covers | clean & lube |
| Lights | test, spare bulbs, corrosion | Repair or replace |
| Gauges | test | Repair or replace |
| Shore power | check line, inverter, sockets, | Repair or replace |
| fuses and breakers | inspect | Replace as needed |
| Connections | Corrosion, shrink wrapped, copper | Clean, replace as needed |
| Wires | neat, organized, labeled | Repair or replace |
| Electronics |  |  |
| Radio | check, spare batteries, check antenna | Repair or replace |
| GPS | check, spare batteries, check antenna | Repair or replace |
| Sonar | check | Repair or replace |
| Radar | check | Repair or replace |
| Compass | check | calibrate |
| Nav | check charts / tide book / nav tools / logbook / radio channel list | replace |
| Head | working, chemicals and TP at hand | clean, lube, treat, replace |
| Trailer |  |  |
| rollers and pads | Worn, loose | Repair or replace |
| wheel bearings | Check | lubricate |
| winch | Check | Clean and lubricate |
| winch pawl / latch | Check | Clean and lubricate |
| winch strap | Check | Clean and lubricate |
| safety chain from boat to trailer | Check | Clean and lubricate |
| tongue jack | Check | Clean and lubricate |
| lights | Check | Clean and lubricate |
| electrical connection | Check | Clean and lubricate |
| tires | pressure and condition | Inflate, service |
| wheels | lug nuts | Tighten, replace |
| brakes | breakaway chains | Repair or replace |
| tongue lock | works | Repair or replace |
| Boat on trailer | Level, tongue weight | Adjust |
| Mainsail | clean, stiff, wear / chafing | Repair or replace |
| Main halyard | Bowline through twist pin U shackle, pins and retaining rings | Repair or replace |
| Roller / furler gear | wear, damage, drum, stay couplings | Repair or replace |
| Reef points | wear, damage | Repair or replace |
| Attachment hardware | wear, damage | Repair or replace |
| Batten pockets - | wear, damage | Repair or replace |
| Battens | wear, damage | Repair or replace |
| Stitching – seams, | wear, damage | Repair or replace |
| Jib | clean, stiff, wear / chafing | Repair or replace |
| Jib halyard | Bowline through twist pin U shackle, pins and retaining rings | Repair or replace |
| Roller / furler gear | wear, damage, tape, drum, stay couplings | Repair or replace |
| Attachment hardware | wear, damage | Repair or replace |
| Stitching – seams | wear, damage | Repair or replace |
| Genoa | clean, stiff, wear / chafing | Repair or replace |
| Genoahalyard | Bowline through twist pin U shackle, pins and retaining rings | Repair or replace |
| Roller / furler gear | wear, damage, tape, drum, stay couplings | Repair or replace |
| Attachment hardware | wear, damage | Repair or replace |
| Stitching – seams | wear, damage | Repair or replace |